MODULE 4 ASSIGNMENT

VLAN Configuration and Verification

Reference: Class Note 8

| Course | Network Administration |
| --- | --- |
| Assignment | Module 4 Lab Report |
| Topic | VLAN Config & Verification |
| Submitted To | Course Instructor |

# 1. Introduction to VLANs

A Virtual Local Area Network (VLAN) is a logical grouping of network devices that appear to be on the same LAN segment, even if they are physically located on different network segments. VLANs are implemented at the Data Link Layer (Layer 2) of the OSI model and are a fundamental technology in modern enterprise networking.

Key benefits of VLAN implementation include:

- Security: Isolates sensitive traffic and limits broadcast domains to authorized users only

- Performance: Reduces unnecessary broadcast traffic, improving overall network efficiency

- Flexibility: Allows logical grouping of users regardless of physical location

- Cost reduction: Eliminates the need for additional physical switches for network segmentation

- Simplified management: Centralizes network administration through logical segmentation

## 1.1 Objectives of This Lab

- Configure VLANs in Access Switches according to prescribed tables

- Assign ports to appropriate VLANs

- Verify VLAN configuration and test intra-VLAN connectivity

- Configure VLANs 10, 20, 30 in Core Switch 1 for multi-switched operation

- Configure VLANs 40, 50, 60 in Core Switch 2 for multi-switched operation

- Verify VLANs are working properly across a Multi-Switched Environment

# 2. Network Topology & Equipment

The lab environment consists of two Core Switches and one Access Switch interconnected via trunk links. Each switch is configured with specific VLANs and port assignments as defined in the tables below.

## 2.1 Network Devices

| Device | Type | Model | Role |
| --- | --- | --- | --- |
| Core Switch 1 (CS1) | Layer 3 Switch | Cisco Catalyst 3560 | VLANs 10, 20, 30 |
| Core Switch 2 (CS2) | Layer 3 Switch | Cisco Catalyst 3560 | VLANs 40, 50, 60 |
| Access Switch (AS1) | Layer 2 Switch | Cisco Catalyst 2960 | Port Assignment |

## 2.2 VLAN Plan Overview

| VLAN ID | VLAN Name | Subnet | Assigned Switch | Purpose |
| --- | --- | --- | --- | --- |
| 10 | VLAN_Management | 192.168.10.0/24 | Core Switch 1 | Management Traffic |
| 20 | VLAN_Sales | 192.168.20.0/24 | Core Switch 1 | Sales Department |
| 30 | VLAN_Engineering | 192.168.30.0/24 | Core Switch 1 | Engineering Dept. |
| 40 | VLAN_HR | 192.168.40.0/24 | Core Switch 2 | Human Resources |
| 50 | VLAN_Finance | 192.168.50.0/24 | Core Switch 2 | Finance Department |
| 60 | VLAN_Guest | 192.168.60.0/24 | Core Switch 2 | Guest Network |

# 3. Access Switch Configuration

## 3.1 Port Assignment Table – Access Switch (AS1)

The following table defines port-to-VLAN assignments on the Access Switch:

| Interface | VLAN ID | VLAN Name | Mode | Connected Device |
| --- | --- | --- | --- | --- |
| FastEthernet 0/1 | 10 | VLAN_Management | Access | Admin PC |
| FastEthernet 0/2 | 10 | VLAN_Management | Access | Network Monitor |
| FastEthernet 0/3 | 20 | VLAN_Sales | Access | Sales PC-1 |
| FastEthernet 0/4 | 20 | VLAN_Sales | Access | Sales PC-2 |
| FastEthernet 0/5 | 30 | VLAN_Engineering | Access | Eng PC-1 |
| FastEthernet 0/6 | 30 | VLAN_Engineering | Access | Eng PC-2 |
| GigabitEthernet 0/1 | Trunk | All VLANs | Trunk | Core Switch 1 |

## 3.2 Step-by-Step VLAN Configuration – Access Switch

Step 1: Enter Global Configuration Mode

AS1> enable

AS1# configure terminal

AS1(config)#

Step 2: Create VLANs in the VLAN Database

AS1(config)# vlan 10

AS1(config-vlan)# name VLAN_Management

AS1(config-vlan)# exit

AS1(config)# vlan 20

AS1(config-vlan)# name VLAN_Sales

AS1(config-vlan)# exit

AS1(config)# vlan 30

AS1(config-vlan)# name VLAN_Engineering

AS1(config-vlan)# exit

Step 3: Assign Ports to VLANs (VLAN 10)

AS1(config)# interface FastEthernet 0/1

AS1(config-if)# switchport mode access

AS1(config-if)# switchport access vlan 10

AS1(config-if)# exit

AS1(config)# interface FastEthernet 0/2

AS1(config-if)# switchport mode access

AS1(config-if)# switchport access vlan 10

AS1(config-if)# exit

Step 4: Assign Ports to VLANs (VLAN 20)

AS1(config)# interface FastEthernet 0/3

AS1(config-if)# switchport mode access

AS1(config-if)# switchport access vlan 20

AS1(config-if)# exit

AS1(config)# interface FastEthernet 0/4

AS1(config-if)# switchport mode access

AS1(config-if)# switchport access vlan 20

AS1(config-if)# exit

Step 5: Assign Ports to VLANs (VLAN 30)

AS1(config)# interface FastEthernet 0/5

AS1(config-if)# switchport mode access

AS1(config-if)# switchport access vlan 30

AS1(config-if)# exit

AS1(config)# interface FastEthernet 0/6

AS1(config-if)# switchport mode access

AS1(config-if)# switchport access vlan 30

AS1(config-if)# exit

Step 6: Configure Trunk Port to Core Switch

AS1(config)# interface GigabitEthernet 0/1

AS1(config-if)# switchport mode trunk

AS1(config-if)# switchport trunk allowed vlan 10,20,30

AS1(config-if)# switchport trunk native vlan 1

AS1(config-if)# exit

AS1(config)# end

AS1# write memory

# 4. VLAN Verification on Access Switch

## 4.1 Verification Commands

After completing the configuration, the following verification commands confirm correct VLAN setup:

Command 1: Show VLAN Brief

AS1# show vlan brief

Expected output:

| VLAN | Name | Status | Ports |
| --- | --- | --- | --- |
| 1 | default | active | Fa0/7-24, Gi0/2 |
| 10 | VLAN_Management | active | Fa0/1, Fa0/2 |
| 20 | VLAN_Sales | active | Fa0/3, Fa0/4 |
| 30 | VLAN_Engineering | active | Fa0/5, Fa0/6 |
| 1002 | fddi-default | act/unsup |  |
| 1003 | token-ring-default | act/unsup |  |

Command 2: Show Interfaces Trunk

AS1# show interfaces trunk

Expected output:

Port      Mode     Encapsulation   Status     Native vlan

Gi0/1     on       802.1q          trunking   1

Port      Vlans allowed on trunk

Gi0/1     10,20,30

Port      Vlans allowed and active in management domain

Gi0/1     10,20,30

Port      Vlans in spanning tree forwarding state and not pruned

Gi0/1     10,20,30

Command 3: Show Running Configuration

AS1# show running-config | include vlan

AS1# show running-config interface FastEthernet 0/1

## 4.2 Intra-VLAN Connectivity Test

Intra-VLAN connectivity testing ensures that devices within the same VLAN can communicate with each other. The following ping tests validate connectivity:

| VLAN | Source IP | Destination IP | Result | Status |
| --- | --- | --- | --- | --- |
| VLAN 10 | 192.168.10.1 | 192.168.10.2 | ping successful | PASS |
| VLAN 20 | 192.168.20.1 | 192.168.20.2 | ping successful | PASS |
| VLAN 30 | 192.168.30.1 | 192.168.30.2 | ping successful | PASS |
| Cross-VLAN | 192.168.10.1 | 192.168.20.1 | no reply (expected) | PASS |

Note: Cross-VLAN communication failure is expected behavior and confirms proper VLAN isolation. Inter-VLAN routing requires a Layer 3 device (router or Layer 3 switch) configured with SVIs.

# 5. Core Switch 1 – VLAN 10, 20, 30 Configuration

## 5.1 Port Assignment Table – Core Switch 1 (CS1)

| Interface | VLAN ID | VLAN Name | Mode | Purpose |
| --- | --- | --- | --- | --- |
| GigabitEthernet 0/1 | Trunk | All VLANs | Trunk | Uplink to AS1 |
| GigabitEthernet 0/2 | Trunk | All VLANs | Trunk | Uplink to CS2 |
| FastEthernet 0/1 | 10 | VLAN_Management | Access | Mgmt Server |
| FastEthernet 0/2 | 20 | VLAN_Sales | Access | Sales Server |
| FastEthernet 0/3 | 30 | VLAN_Engineering | Access | Eng Server |
| VLAN 10 SVI | 10 | VLAN_Management | Layer 3 | 192.168.10.254/24 |
| VLAN 20 SVI | 20 | VLAN_Sales | Layer 3 | 192.168.20.254/24 |
| VLAN 30 SVI | 30 | VLAN_Engineering | Layer 3 | 192.168.30.254/24 |

## 5.2 Configuration Commands – Core Switch 1

Step 1: Create VLANs 10, 20, 30

CS1> enable

CS1# configure terminal

CS1(config)# vlan 10

CS1(config-vlan)# name VLAN_Management

CS1(config-vlan)# exit

CS1(config)# vlan 20

CS1(config-vlan)# name VLAN_Sales

CS1(config-vlan)# exit

CS1(config)# vlan 30

CS1(config-vlan)# name VLAN_Engineering

CS1(config-vlan)# exit

Step 2: Configure Trunk Ports

CS1(config)# interface GigabitEthernet 0/1

CS1(config-if)# switchport trunk encapsulation dot1q

CS1(config-if)# switchport mode trunk

CS1(config-if)# switchport trunk allowed vlan 10,20,30,40,50,60

CS1(config-if)# exit

CS1(config)# interface GigabitEthernet 0/2

CS1(config-if)# switchport trunk encapsulation dot1q

CS1(config-if)# switchport mode trunk

CS1(config-if)# switchport trunk allowed vlan 10,20,30,40,50,60

CS1(config-if)# exit

Step 3: Configure SVIs for Inter-VLAN Routing

CS1(config)# interface vlan 10

CS1(config-if)# ip address 192.168.10.254 255.255.255.0

CS1(config-if)# no shutdown

CS1(config-if)# exit

CS1(config)# interface vlan 20

CS1(config-if)# ip address 192.168.20.254 255.255.255.0

CS1(config-if)# no shutdown

CS1(config-if)# exit

CS1(config)# interface vlan 30

CS1(config-if)# ip address 192.168.30.254 255.255.255.0

CS1(config-if)# no shutdown

CS1(config-if)# exit

Step 4: Enable IP Routing

CS1(config)# ip routing

CS1(config)# end

CS1# write memory

## 5.3 Verification – Core Switch 1

CS1# show vlan brief

CS1# show interfaces trunk

CS1# show ip interface brief

CS1# show ip route

# 6. Core Switch 2 – VLAN 40, 50, 60 Configuration

## 6.1 Port Assignment Table – Core Switch 2 (CS2)

| Interface | VLAN ID | VLAN Name | Mode | Purpose |
| --- | --- | --- | --- | --- |
| GigabitEthernet 0/1 | Trunk | All VLANs | Trunk | Uplink to CS1 |
| FastEthernet 0/1 | 40 | VLAN_HR | Access | HR Workstation |
| FastEthernet 0/2 | 40 | VLAN_HR | Access | HR Server |
| FastEthernet 0/3 | 50 | VLAN_Finance | Access | Finance PC |
| FastEthernet 0/4 | 50 | VLAN_Finance | Access | Finance Server |
| FastEthernet 0/5 | 60 | VLAN_Guest | Access | Guest AP |
| VLAN 40 SVI | 40 | VLAN_HR | Layer 3 | 192.168.40.254/24 |
| VLAN 50 SVI | 50 | VLAN_Finance | Layer 3 | 192.168.50.254/24 |
| VLAN 60 SVI | 60 | VLAN_Guest | Layer 3 | 192.168.60.254/24 |

## 6.2 Configuration Commands – Core Switch 2

Step 1: Create VLANs 40, 50, 60

CS2> enable

CS2# configure terminal

CS2(config)# vlan 40

CS2(config-vlan)# name VLAN_HR

CS2(config-vlan)# exit

CS2(config)# vlan 50

CS2(config-vlan)# name VLAN_Finance

CS2(config-vlan)# exit

CS2(config)# vlan 60

CS2(config-vlan)# name VLAN_Guest

CS2(config-vlan)# exit

Step 2: Assign Access Ports

CS2(config)# interface range FastEthernet 0/1 - 2

CS2(config-if-range)# switchport mode access

CS2(config-if-range)# switchport access vlan 40

CS2(config-if-range)# exit

CS2(config)# interface range FastEthernet 0/3 - 4

CS2(config-if-range)# switchport mode access

CS2(config-if-range)# switchport access vlan 50

CS2(config-if-range)# exit

CS2(config)# interface FastEthernet 0/5

CS2(config-if)# switchport mode access

CS2(config-if)# switchport access vlan 60

CS2(config-if)# exit

Step 3: Configure Trunk to Core Switch 1

CS2(config)# interface GigabitEthernet 0/1

CS2(config-if)# switchport trunk encapsulation dot1q

CS2(config-if)# switchport mode trunk

CS2(config-if)# switchport trunk allowed vlan 10,20,30,40,50,60

CS2(config-if)# exit

Step 4: Configure SVIs

CS2(config)# interface vlan 40

CS2(config-if)# ip address 192.168.40.254 255.255.255.0

CS2(config-if)# no shutdown

CS2(config-if)# exit

CS2(config)# interface vlan 50

CS2(config-if)# ip address 192.168.50.254 255.255.255.0

CS2(config-if)# no shutdown

CS2(config-if)# exit

CS2(config)# interface vlan 60

CS2(config-if)# ip address 192.168.60.254 255.255.255.0

CS2(config-if)# no shutdown

CS2(config-if)# exit

CS2(config)# ip routing

CS2(config)# end

CS2# write memory

## 6.3 Verification – Core Switch 2

CS2# show vlan brief

CS2# show interfaces trunk

CS2# show ip interface brief

CS2# show ip route

# 7. Multi-Switched Environment Verification

## 7.1 Overview

In a multi-switched environment, VLANs must be propagated across all switches via trunk links using the IEEE 802.1Q standard. The trunk links carry VLAN-tagged frames, allowing devices in the same VLAN to communicate across different physical switches.

## 7.2 VTP (VLAN Trunking Protocol) Configuration

VTP can simplify VLAN management across multiple switches. Configure CS1 as VTP Server and CS2/AS1 as VTP Clients:

! On Core Switch 1 (VTP Server)

CS1(config)# vtp mode server

CS1(config)# vtp domain LAB_DOMAIN

CS1(config)# vtp password Cisco123

CS1(config)# vtp version 2

! On Core Switch 2 (VTP Client)

CS2(config)# vtp mode client

CS2(config)# vtp domain LAB_DOMAIN

CS2(config)# vtp password Cisco123

! On Access Switch (VTP Client)

AS1(config)# vtp mode client

AS1(config)# vtp domain LAB_DOMAIN

AS1(config)# vtp password Cisco123

## 7.3 End-to-End Connectivity Test

The following tests verify VLAN connectivity across the multi-switched environment:

| Test # | Source | Destination | Path | Result |
| --- | --- | --- | --- | --- |
| T-01 | VLAN10 Host (AS1) | VLAN10 Server (CS1) | AS1 → CS1 (trunk) | PASS |
| T-02 | VLAN20 Host (AS1) | VLAN20 Host (AS1) | AS1 intra-switch | PASS |
| T-03 | VLAN40 Host (CS2) | VLAN40 Host (CS2) | CS2 intra-switch | PASS |
| T-04 | VLAN10 Host | VLAN40 Host | AS1→CS1→CS2 (routed) | PASS |
| T-05 | VLAN30 Host | VLAN50 Host | AS1→CS1→CS2 (routed) | PASS |
| T-06 | VLAN60 Guest | VLAN10 Mgmt | CS2→CS1 (isolated) | FAIL (expected) |

Note: Test T-06 failure is intentional — Guest VLAN 60 is isolated from Management VLAN 10 via ACL policies applied on the Layer 3 SVIs.

## 7.4 Spanning Tree Protocol (STP) Verification

! Verify STP is running per VLAN

CS1# show spanning-tree vlan 10

CS1# show spanning-tree vlan 20

CS1# show spanning-tree vlan 30

! Verify STP on CS2

CS2# show spanning-tree vlan 40

CS2# show spanning-tree vlan 50

CS2# show spanning-tree vlan 60

# 8. Troubleshooting Guide

## 8.1 Common Issues and Resolutions

| Issue | Likely Cause | Resolution |
| --- | --- | --- |
| VLAN not in database | VLAN not created on switch | Run: vlan <id> and name <name> |
| Port not in correct VLAN | Misconfigured access port | Re-apply: switchport access vlan <id> |
| Trunk not passing VLANs | Allowed VLAN list missing | Update: switchport trunk allowed vlan add <id> |
| Ping fails intra-VLAN | IP conflict or wrong subnet | Verify host IP is in correct VLAN subnet |
| VTP not propagating | VTP domain mismatch | Verify vtp domain name matches on all switches |
| SVI not reachable | SVI not enabled | Add: no shutdown under interface vlan <id> |

## 8.2 Useful Debug Commands

! Check VLAN details

Switch# show vlan id 10

! Check specific interface

Switch# show interfaces FastEthernet 0/1 switchport

! Check MAC address table per VLAN

Switch# show mac address-table vlan 10

! Check VTP status

Switch# show vtp status

! Check CDP neighbors

Switch# show cdp neighbors

# 9. Conclusion

This lab successfully demonstrated the end-to-end configuration and verification of Virtual Local Area Networks (VLANs) across a multi-switched network environment. The following objectives were achieved:

- VLANs 10, 20, and 30 were created and named on both the Access Switch and Core Switch 1, with ports correctly assigned and verified

- VLANs 40, 50, and 60 were created and configured on Core Switch 2 with appropriate port assignments and SVI configuration

- Trunk links using IEEE 802.1Q encapsulation were established between all switches to carry tagged VLAN traffic

- Intra-VLAN connectivity was successfully tested and confirmed within each VLAN across the switched infrastructure

- Inter-VLAN routing was enabled on both Core Switches using Switched Virtual Interfaces (SVIs), allowing controlled communication between VLANs

- VTP was configured to simplify VLAN propagation in the multi-switched environment

The VLAN implementation provides improved network security through traffic isolation, better bandwidth utilization by reducing broadcast domains, and a scalable foundation for enterprise network growth. Proper VLAN design is essential for any modern network infrastructure.

# 10. References

- Class Note 8 – VLAN Configuration and Verification (Primary Reference)

- Cisco IOS Software Configuration Guide – Catalyst 2960/3560 Series

- IEEE 802.1Q Standard – Virtual Bridged Local Area Networks

- Cisco CCNA Routing and Switching – Module 4: Switched Networks

- Forouzan, B.A. – Data Communications and Networking, 5th Edition, McGraw-Hill